# Class D Audio Amplifier (Proteus/KiCad)

## Overview

A Class D amplifier is a switching audio amplifier that converts the analog input into a high-frequency pulse-train whose duty cycle encodes the audio amplitude. The output transistors (MOSFETs) are driven fully on/off, so they dissipate very little power (unlike linear amps), yielding theoretical efficiencies near 100% (practically ≥90%). In this design, a 1 kHz sine input (0.5 V peak) is compared with a ~25 kHz triangle wave to generate a PWM signal (duty ∝ audio). The PWM drives a half-bridge MOSFET stage, and an LC low-pass filter recovers the amplified audio. Class D was chosen for its high efficiency (suitable for battery/heat-limited use) and because the design can be implemented with open-source tools (Proteus simulation and KiCad PCB).

## Components List

The key parts (values and example packaging) are:

- ICs: LM358 (Dual op-amp, e.g. TI LM358 in SOIC-8 SMD or PDIP-8), LM311 (Comparator, DIP-8 or SOIC-8). 7805 (5 V regulator, TO-220).

- Transistors: Q1 = BC547 NPN (TO-92 through-hole), Q2 = BC557 PNP (TO-92).

- MOSFETs: Q3 = IRF9540N P-channel MOSFET (TO-220), Q4 = IRF540N N-channel MOSFET (TO-220). These handle the 24 V switching.

- Resistors: R1,R2,R3,R6,R7 = 10 kΩ; R4,R5 = 1 kΩ (all 0805 SMD or ¼W THT as needed). R<sub>LOAD</sub> = 8 Ω (speaker or resistor).

- Capacitors: C1 = 1 µF (electrolytic or film, 25 V), C2 = 1 nF (ceramic), C3 = 100 pF (NP0 ceramic), C4 = 2.2 µF (electrolytic or film, 35 V). Plus bypass caps for 7805: 0.33 µF (ceramic) at input, 0.1 µF (ceramic) at output, and 10 µF electrolytic at both input/output.

- Inductor: L1 = 100 µH power inductor (through-hole ferrite type).

- Connectors: DC barrel jack (5.5×2.1 mm, center positive) for 12–24 V input, and a 3.5 mm audio jack (mono) for signal input.

- PCB: 21 mm × 65 mm board (two-sided). Major power components (MOSFETs, regulator) are THT for heat-sinking; smaller parts (resistors, caps, op-amps) can be SMD. A 3D preview (KiCad) shows the mixed SMD/THT layout and tight size.

## System Block Diagram and Explanation

Figure: Simplified half-bridge Class D amplifier block diagram (audio → comparator (PWM) → MOSFET driver → LC filter). This illustrates the stages of our design.

The system is divided into functional blocks:

- Block 1: Audio Input. This stage provides the input signal to be amplified. We use a 1 kHz sine-wave source (VSIN, 0.5 V amplitude) AC-coupled through R1 (10 kΩ) and C1 (1 µF). C1 and R1 form a high-pass filter (f<sub>c</sub>≈16 Hz) to block any DC and set the input impedance. The resistor also biases the input for single-supply operation. This prepares the audio for PWM comparison.

- Block 2: Triangle-Wave Generator. An op-amp (LM358) is configured to generate a high-frequency triangle (ramp) waveform ~25 kHz. With R2=10 kΩ, R3=10 kΩ and C2=1 nF, the op-amp oscillates (Schmitt trigger + integrator) to produce a near-triangular wave. This high-frequency ramp is used as the PWM carrier. (Any precision single-supply op-amp is suitable; LM358 is chosen here for its low cost and single-supply range.)

- Block 3: PWM Comparator. A comparator (LM358 or LM311) takes the audio (+IN) and the triangle wave (–IN). When the audio exceeds the triangle, the comparator output goes high, otherwise low, producing a PWM waveform ~25 kHz whose duty cycle is proportional to the audio. In other words, the audio modulates the on-time of the output transistor switches. The PWM waveform swings between the logic rails and drives the MOSFET gates.

- Block 4: Dead-Time & Gate Driver. To safely drive the half-bridge, we use a simple transistor gate-driver with built-in dead-time. Two bipolar transistors (Q1=BC547 NPN, Q2=BC557 PNP) form a totem-pole driver that inverts one gate signal and buffers it into push-pull outputs. Small series resistors R4,R5=1 kΩ limit the base drive, and pull-ups R6,R7=10 kΩ bias the gates. A small capacitor C3=100 pF adds a slight RC delay on one side, ensuring that when PWM changes state, one MOSFET gate is delayed slightly. This “dead-time” prevents both MOSFETs from conducting simultaneously (shoot-through) during switching. In summary, the gate-driver alternately turns on Q3 (high-side) or Q4 (low-side) MOSFET according to the PWM, with a brief overlap delay.

- Block 5: Power Stage (Half-Bridge). The high-power switches are Q3 = IRF9540N (P-channel, for high-side) and Q4 = IRF540N (N-channel, low-side), forming a half-bridge powered from +24 V. When the PWM is high, the P-channel MOSFET (Q3) turns on and connects the output node to +24 V; when low, the N-channel MOSFET (Q4) connects it to ground. Thus the output node switches between 0 V and 24 V at ~25 kHz (50% duty at quiescent), producing a high-frequency square-wave whose average is the audio. IRF9540N/IRF540N are chosen for their low R<sub>DS(on)</sub> and adequate voltage/current ratings (TO-220 package for heat sinking).

- Block 6: LC Output Filter. We use an LC low-pass filter (L1 = 100 µH, C4 = 2.2 µF) followed by the 8 Ω load. The purpose is to remove the 25 kHz carrier and pass only the audio (<20 kHz) to the speaker. An LC filter provides a steep cutoff and low loss compared to a simple RC, since the inductor stores and reuses energy. (For example, TI’s filter-design equations would suggest ~47 µH + 0.47 µF for fc ≈34 kHz; our 100 µH/2.2 µF yields a similar order-of-magnitude cutoff well above 20 kHz.) The filter behaves as a passive integrator: its output is essentially the average (fundamental) value of the PWM waveform, reconstructing the amplified audio sine wave.

## 🔁 Simple Working Flow (One-Line)

Potentiometer (Audio Level) → Comparator + Triangle Wave → PWM Signal → MOSFET Half-Bridge → LC Filter → Speaker Output

## Simulation (Proteus)

Proteus was used to validate the logic and switching behavior. The figures below illustrate typical waveforms at each stage (as also described in Analog Devices’ class-D reference).

Figure: Simulated PWM output from the comparator (0–24 V square wave at ~25 kHz). The pulse width varies with the input sine amplitude (duty ∝ input).

- Audio Input (Block 1): A 1 kHz, 0.5 V (p-p) sine wave is fed through R1/C1 into the comparator.

- Comparator Output (PWM, Block 3): A ~25 kHz square wave (0–12V) whose duty cycle tracks the audio. As in Figure above, a larger input sine amplitude produces a wider pulse. At zero audio, the duty is ~50% (midpoint).

- Half-Bridge Output (MOSFET Drains, Block 5): The drain node rapidly switches between + V and 0 V. For example, when PWM is high, the high-side MOSFET pulls the node to +12 V; when low, the low-side pulls it to 0. This creates the high-frequency carrier.

- Filter Output (Block 6): After L1–C4, the waveform is a clean 1 kHz sine (scaled by the circuit gain). The LC filter effectively averages the PWM, recovering the audio. The speaker (8 Ω) sees the amplified audio with negligible 25 kHz ripple.

## PCB Design (KiCad)

The schematic was laid out in KiCad and a 2-layer PCB created (top + bottom copper). A ground pour was used on the bottom to minimize loop area. High-current traces (24 V rail, MOSFET gate drive) were made wider (~1–1.5 mm) for lower resistance, while signal traces (audio, op-amps) use standard 0.3–0.5 mm. The dimensions are 21×65 mm, fitting easily in a compact enclosure. The DC barrel jack footprint is placed at one edge for the 12–24 V supply, and the 3.5 mm audio jack at another edge. Mixed SMD/THT placement was used: the power components (IRF540/9540, 7805, input jack) are through-hole with solder lugs (and can use a heatsink), while most resistors, caps, and the LM358/LM311 are SMD to save space. A KiCad 3D render (not shown) confirms that all parts fit within the small board size.

THANK YOU